AC Current Isolation from Total Load Signal

ML Internship Assignment — Electrical Data Analysis

Data: 22–23 June 2024  |  5,575 samples  |  ~24 hours  |  Voltage: ~240–253 V

# 1. Data Overview

The dataset captures one full day of electrical readings from a single room, sampled approximately every 2 minutes. Each record contains a Unix millisecond timestamp, voltage (V), and total room current (A).

- 5,575 total samples spanning 22 Jun 18:30 to 23 Jun 18:28 (~24 hours)

- Voltage ranges 240–253 V — standard Indian residential single-phase supply

- Current spans 0.02 A to 25.4 A — a ~1,000× dynamic range indicating multiple load types

- Mean current = 1.78 A, but median = only 0.09 A — indicating most readings are low-load background

# 2. Full 24-Hour Current Signal

The plot below shows the total current over the entire day. AC-ON periods (above the 5 A threshold) are highlighted in red; background load periods are in blue.

Fig. 1: 24-hour current signal resampled to 2-min medians. Red fill = AC ON; blue fill = background. Dashed line = 5 A threshold.

# 3. Current Distribution — Bimodal Separation

The histogram below confirms the key finding: there is a near-empty gap between ~0.4 A and ~8 A. The two load regimes are completely separable by a simple threshold.

Fig. 2: Left — background appliance current clusters tightly below 0.4 A. Right — AC current clusters around 9–12 A (inverter modulation). The yellow dashed line marks the mean AC current (~10.5 A).

This bimodal structure is the physical basis of the isolation approach. A non-inverter AC would show a hard on/off step; the smooth unimodal cluster seen here confirms an inverter-type compressor that modulates speed continuously.

# 4. AC Usage Pattern by Hour

The charts below show when during the day AC was active, and how power consumption breaks down by load type across each hour.

Fig. 3: Left — % of readings with AC ON per hour. Right — avg power split by load type. Hours 18–21 (evening arrival) and 02–04 (peak night heat) show strongest AC activity.

Usage is consistent with a June summer day in India: the room is unoccupied or naturally ventilated during the day, with heavy cooling needed in the evening and again late at night when outdoor temperatures peak.

# 5. Proposed Method to Isolate AC Consumption

## Step 1 — Threshold Segmentation

Apply a hard threshold at 5 A. The wide gap in the distribution (0.4 A to 8 A) gives ample safety margin:

- AC current = total current   if   I > 5 A

- Background = total current   if   I ≤ 5 A

## Step 2 — Background Baseline Estimation

During AC-ON periods, background loads (fans, lights) are still running but hidden under the AC signal. To cleanly remove them:

- Compute hourly median of current during AC-OFF windows for each hour of the day

- Interpolate / forward-fill this baseline into AC-ON time slots

- Isolated AC current = total current − estimated background baseline

## Step 3 — Power Conversion & Smoothing

- Multiply isolated current × voltage to get AC power in Watts

- Apply a 3–5 min rolling median to smooth inverter micro-fluctuations

- Flag startup spikes (I > 18 A, duration < 30 s) separately as compressor inrush

# 6. Isolated AC Signal (After Background Subtraction)

The two-panel chart below shows the before (raw total current) and after (isolated AC current) of the proposed method applied to the dataset.

Fig. 4: Top panel — raw total current with AC-ON regions. Bottom panel — isolated AC current after subtracting hourly background baseline. Green dotted line = interpolated background estimate.

# 7. Key Metrics Summary

| Metric | Value |
| --- | --- |
| Data Period | 22 Jun 2024 18:30 – 23 Jun 2024 18:28 (24 hrs) |
| Total Readings | 5,575 samples |
| AC Avg Current | ~10.5 A |
| AC Avg Power | ~2,617 W (2.6 kW) |
| Background Avg Current | ~0.15 A (lights, fans, etc.) |
| Background Avg Power | ~37.6 W |
| AC Active (% of day) | ~15.7% of total readings |
| Proposed AC Threshold | > 5 A (clear bimodal gap) |
| AC Type Estimate | Inverter (unimodal, low CV) |
| Likely AC Size | 1.5–2 ton (~1,500–2,000 W nameplate) |

# 8. Conclusion

The total current signal contains a naturally separable AC signature. The ~70× magnitude difference between AC operation (~10.5 A) and background loads (~0.15 A) creates a clean bimodal distribution with virtually no overlap, making a 5 A threshold a highly reliable separator.

The smooth, unimodal AC current cluster (low coefficient of variation) confirms an inverter-type compressor — an important detail, as a non-inverter AC would require a different detection approach. Usage timing (evening + late-night peaks) aligns with June summer occupancy patterns in India.

The resulting isolated AC time-series — produced without any model training — is directly suitable as a labelled feature for downstream ML development.

orion-blue  |  ML Internship Assignment  |  June 2024 Data